Q1 — “Explain your project in simple terms”

✅ Answer:

“This project develops a dynamic congestion pricing system for Auckland. Traffic data is generated using SUMO simulation and processed into corridor-level travel time. Machine learning models such as ARIMA, XGBoost, and LSTM are used to predict short-term traffic conditions. Based on predicted congestion, a pricing algorithm dynamically adjusts tolls. The system also models how drivers respond to pricing and evaluates how this affects congestion. Finally, MQTT is proposed for real-time implementation using IoT data.”

❓ Q2 — “How do we know your data is correct?”

✅ Answer:

“The data pipeline is fully traceable. It starts from SUMO simulation outputs, which are converted into structured CSV data, then aggregated into 10-minute travel time. Each transformation step is validated, and the final dataset used for modelling is consistent and reproducible.”

❓ Q3 — “Why did you aggregate to 10 minutes?”

✅ Answer:

“Aggregation to 10-minute intervals reduces noise and creates a stable time-series suitable for forecasting models. It also aligns with short-term traffic prediction use cases in transport systems.”

❓ Q4 — “Why 80–20 split?”

✅ Answer:

“An 80–20 chronological split ensures sufficient data for training while preserving unseen future data for evaluation. This maintains the temporal integrity of the time-series.”

❓ Q5 — “Why ARIMA performed as good as LSTM?”

✅ Answer:

“Because the data is aggregated at corridor level, the signal becomes smooth and highly autocorrelated. This reduces the advantage of complex models like LSTM, allowing ARIMA to perform equally well.”

❓ Q6 — “How did you calculate congestion?”

✅ Answer:

“Congestion is measured using a congestion index defined as the ratio of predicted travel time to free-flow travel time. Free-flow is estimated using the 5th percentile of observed travel times.”

❓ Q7 — “Why 5th percentile?”

✅ Answer:

“The 5th percentile provides a robust estimate of uncongested conditions while avoiding extreme outliers that may occur with minimum values.”

❓ Q8 — “Explain your pricing model”

✅ Answer:

“The pricing model is based on congestion levels. When the congestion index exceeds a threshold, tolls increase proportionally, with a cap to ensure fairness and policy realism.”

❓ Q9 — “Is your system realistic?”

✅ Answer:

“Yes, the system is realistic in structure. While the current implementation uses simulation data, it can be directly extended to real-time deployment using IoT data via MQTT.”

❓ Q10 — “Why should we trust your results?”

✅ Answer:

“The results are based on a structured pipeline, validated datasets, and standard evaluation metrics such as MAE and RMSE. Additionally, the system behaviour aligns with known traffic dynamics, such as demand reduction under pricing.”

❓ Q11 — “What is your main contribution?”

✅ Answer:

“The main contribution is an integrated framework combining traffic simulation, AI-based forecasting, dynamic pricing, behavioural modelling, and real-time system design in a single pipeline.”

❓ Q12 — “What are your limitations?”

✅ Answer:

“The system currently uses simulated data and assumes fixed elasticity. Real-world deployment would require calibration using real traffic data and more advanced behavioural modelling.”

❓ Q13 — “How does MQTT fit in your system?”

✅ Answer:

“MQTT enables real-time communication between traffic sensors and the prediction system. It allows live data streaming, which can replace simulation inputs in a real deployment.”